Question

I used to work with Gemini 3 for the following project, but the results were bad, and I went nowhere. Could you have me make a realistic plan?

I am a college student, and this is a new project in which I want to develop a car parking algorithm. Because I have a 2018 Honda Pilot, which does not have a rearward microwave distance detection system, it only has a rear camera.  So, I bought four microwave detectors for the car and installed them, which will alert me when I back up. Taking a cue from Tesla's auto parking system, I'd like to develop a similar parking system that uses a camera to detect parking lines and build a simulation to practice auto parking.

Using a real camera to look at the back side, identify the parking lines.

Look for an existing parking lane detection model.

Find a suitable simulation environment—or build one myself—to develop the parking algorithm. For example, how to move a rectangular box into a parking position.

Simulate microwave-based distance detection to provide alerts for rear-boundary obstacles.

I'd like to hear your suggestion on how to complete my project. Thank you

## 0. Decide the Scope (very important)

Before anything, lock in:

- Parking type: Start with simple back-in perpendicular parking into a single spot in an empty lot.

- Goal for this project:

  - ✅ Get a 2D simulation where a car-shaped box auto-parks into a marked slot using:

    - Parking-line information (from synthetic or real images)

    - Simulated “microwave” distance sensors

  - 🔁 Optional stretch: connect it later to your real car sensors/camera.

If you try to do:

- full 3D physics

- perfect real-world vision

- full Tesla-style robustness

…you will stall again. So: simulation first, reality later.

## 1. Phase 1 – Minimal Simulation Environment (2–3 weeks)

Goal: A very simple 2D world where a rectangle (“car”) moves and rotates.

Tools (pick ONE simple stack):

- Language: Python

- Libraries:

  - matplotlib or pygame for drawing

  - OR just use a Jupyter notebook with a simple plot

What to implement:

- Car model (kinematics, not full physics):

  - State: ((x, y, \theta)) – center position + heading.

  - Inputs: speed (v) (forward/backward), steering angle (\delta).

  - Simple “bicycle model” update each timestep:

    - (x_{t+1} = x_t + v \cos\theta \cdot \Delta t)

    - (y_{t+1} = y_t + v \sin\theta \cdot \Delta t)

    - (\theta_{t+1} = \theta_t + \frac{v}{L} \tan\delta \cdot \Delta t)
(where (L) is wheelbase)

- Parking spot & environment:

  - Represent the parking spot as a rectangle in world coordinates.

  - Draw parking lines as simple line segments.

  - No obstacles yet.

- Keyboard/manual control (tiny but useful):

  - Let yourself press keys (e.g., left/right/up/down) to move the car.

  - This helps you debug and visualize before any “auto” logic.

Deliverable at end of Phase 1:

- A script where you:

  - See a top-down view

  - Can manually drive a rectangle into a parking box

  - Confirm the “car model” and drawing work.

## 2. Phase 2 – Simulated Microwave Distance Sensors (1 week)

You already installed real sensors on your car, but for now we’ll fake them in the sim.

What to do:

- Imagine 4 sensors on the car’s rear bumper:

  - Left, mid-left, mid-right, right.

  - Each casts a ray backwards in the world.

- In the simulation:

  - For each sensor, cast a ray and compute distance to the nearest obstacle (back boundary wall, other car, etc.).

  - If distance < threshold → trigger alert (e.g., print “BEEP” or color the sensor red).

- Add one rear obstacle (like a wall) in the environment rectangle.

Deliverable:

- When you reverse the car manually, the simulated sensors show distance readings or warnings:

  - This mimics your microwave sensors on the real car.

  - No autopark yet; just sensor simulation.

## 3. Phase 3 – Parking Path Planning (2–3 weeks)

Now we teach the rectangle how to park by itself into a known spot.

At this stage, assume you already know:

- Where the parking spot is (its pose in the world).

We’ll add camera/vision later; first make the car behave.

### 3.1. Simplest “scripted” parking maneuver

For back-in perpendicular parking, you can:

- Define a target pose:

  - Center of parking spot

  - Desired final heading (straight in)

- Define a sequence of motion primitives, e.g.:

  - Step 1: Pull forward a bit to align

  - Step 2: Reverse with constant steering angle until near spot

  - Step 3: Straighten wheels and keep reversing until inside box

You can hard-code a simple algorithm like:

- If car is left of spot → turn right while backing until aligned

- Then straighten and continue backing.

### 3.2. Controller (keep it simple)

Instead of fancy control, use a very simple PID-like rule:

- Compute angle error = difference between car heading and desired heading.

- Use that to set steering angle.

- Use a fixed reverse speed.

You can iterate:

- Start from a few fixed starting positions.

- Tune parameters so the car ends up inside the spot without collisions.

Use your simulated sensors:

- If any rear sensor < safe distance → stop movement (like real car beeping).

Deliverable:

- From a reasonable starting pose, your sim can:

  - Auto-reverse into the parking spot

  - Stop safely using simulated sensor distances

  - No camera yet, but behavior looks like basic auto-park.

## 4. Phase 4 – Parking Line / Spot Detection from Camera (3–4 weeks)

Only once the parking algorithm works in 2D should you connect vision.

You have two paths here:

### Option A – Classical Computer Vision (recommended to start)

Use OpenCV on either:

- Real rear-camera frames from your Honda (recorded video), or

- Synthetic images you render from a simple 3D scene / toy setup.

Basic pipeline idea:

- Pre-processing:

  - Undistort / crop to region near the ground

  - Maybe convert to HSV and threshold for white/yellow lines

- Edge & line detection:

  - Canny edge detector

  - Hough transform to detect line segments

- Inverse Perspective Mapping (bird’s-eye view):

  - Use a homography to transform the ground plane region into a top-down view.

  - In the bird’s-eye view, parking lines become nice straight lines.

- Detect parking spot from lines:

  - Try to find pairs of roughly parallel lines with a certain spacing.

  - Use that to estimate a parking box in world (x, y, θ).

- Feed detected pose into your planner (from Phase 3):

  - Instead of giving the spot coordinates by hand, compute them from the image.

### Option B – Pre-trained Deep Learning Model (stretch)

If you want to be fancier:

- Look for existing parking slot detection models (e.g., YOLO-based models trained on parking lot datasets).

- But this adds:

  - Dataset

  - Training/inference pipeline

  - More complexity

For a college project, I’d strongly recommend:

Start with simple OpenCV + geometry and only move to DL if you have time.

Deliverable:

- Given a frame from your camera, your code estimates:

  - Approximate position + orientation of a parking spot

- You can visualize:

  - draw a rectangle on the image showing the detected spot

- Then feed that pose into your simulation’s auto-parking algorithm.

## 5. Phase 5 – Integrate Everything (2–3 weeks)

Now connect the pieces:

- Vision → Parking Spot Pose:

  - From camera frame, compute parking spot pose in some coordinate frame.

- Parking Algorithm → Control:

  - Use the pose as the target for your planner (from Phase 3).

  - Generate control commands (v, δ) over time.

- Simulation Loop:

  - At each time step:

    - Apply (v, δ) to update car state.

    - Update sensor distances, check for collision / beeps.

    - Draw everything.

- Demonstration scenarios:

  - Scenario 1: Empty lot, single back-in spot, no obstacles.

  - Scenario 2 (stretch): Add another car behind or to the side as an obstacle; make sure you never collide (thanks to sensors).

Deliverables:

- A short demo video:

  - Show camera frame with detected spot

  - Show 2D simulation of the car auto-parking into the spot

  - Show sensor readings/alerts as the car gets close to the boundary.

## 6. (Optional) Phase 6 – Towards the Real Car

Only if you have time and feel safe doing it.

Ideas:

- Use a laptop + USB capture for the backup camera video.

- Read your real microwave sensor outputs via a microcontroller (Arduino, STM32, etc.) and stream them to the laptop.

- Run a limited version of your algorithm where:

  - You still drive manually, but:

    - System detects lines and suggests steering

    - System warns when you’re too close.

But even without this, a solid simulation + vision pipeline already makes a fantastic project.

## 7. Suggested Timeline (example ~12–14 weeks)

You can adjust, but something like:

- Weeks 1–2: Phase 1 – Build 2D sim + manual driving

- Week 3: Phase 2 – Simulated microwave sensors + obstacles

- Weeks 4–6: Phase 3 – Auto-parking planner & controller in 2D

- Weeks 7–10: Phase 4 – Vision (OpenCV) and parking spot detection

- Weeks 11–12: Phase 5 – Full integration + demo video

- Weeks 13–14 (optional): Phase 6 – Real hardware integration

## 8. What you should work on first (concrete next steps)

If you want a very immediate checklist:

This week:

- Pick:

  - Language: Python

  - Library: matplotlib or pygame for plotting.

- Implement:

  - Simple car state update (x, y, θ).

  - A drawing function that shows a rectangle at that pose.

- Add:

  - Manual control via keyboard.

  - A drawn parking box as a target.

Once you have that running, send me:

- A screenshot of your 2D world

- The core part of your simulation code

And I can help you:

- Clean it up

- Add the sensor model

- Design the parking maneuver.

If you’d like, I can also draft a tiny technical report outline for this project (intro, method, experiments, results) so it doubles as a nice portfolio piece.

Absolutely — this is a great moment to consolidate what you learned.
Below is a clean, structured summary of the key questions you hit during debugging, why they happened, and the solutions you applied, written so you could almost drop this into a project log or report.

# 📘 Parking Lane Detection – Debugging Q&A Summary

This project uses a classical computer vision pipeline (Canny + Hough + geometry) to detect parking lanes from a rear camera image. During development, several recurring issues appeared. Each one taught an important lesson about real-world vision systems.

## Q1. “Hough detects many lines that are NOT parking lanes. How do I remove single/random lines?”

### ❌ Problem

- HoughLinesP detects any straight edge:

  - cracks

  - curb edges

  - texture boundaries

- Parking lanes are not just “lines” — they are two parallel lines with a fixed gap.

### ✅ Solution

Introduce a pair-based geometric constraint:

- Represent each detected segment as an infinite line:
[
ax + by + c = 0
]

- Keep a line only if it belongs to a pair that satisfies:

  - Nearly parallel
[
|\theta_1 - \theta_2| \le \text{angle_pair_th}
]

  - Plausible spacing
[
\text{min_pair_gap} \le |c_1 - c_2| \le \text{max_pair_gap}
]

### 🔑 Insight

Parking lanes are structures, not features.
Filtering by relationships (parallel + gap) is far more powerful than filtering by appearance alone.

## Q2. “After adding pair filtering, nothing is detected at all.”

### ❌ Problem

Initial defaults were too strict:

- High minLineLength

- Tight angle_pair_th

- Narrow min_pair_gap / max_pair_gap

All valid pairs were eliminated before pairing even happened.

### ✅ Solution

Use “early detection” parameters during debugging:

threshold     = low

minLineLength = low

angle_pair_th = large (20–30°)

min_pair_gap  = very small

max_pair_gap  = very large

Then tighten gradually.

### 🔑 Insight

In classical CV:

Always start permissive → then constrain.

If early stages are too strict, later logic never gets a chance.

## Q3. “What exactly does angle_pair_th mean?”

### ❓ Question

What does angle_pair_th physically represent?

### ✅ Explanation

- Each line has an angle from atan2(dy, dx)

- angle_pair_th is the maximum allowed angle difference for two lines to be considered parallel

Example:

- Line A: 88°

- Line B: 91°

- Difference = 3°

| angle_pair_th | Result |
| --- | --- |
| 30° | Accept |
| 10° | Accept |
| 4° | Accept |
| 3° | Reject |

### 🔑 Insight

This parameter controls parallelism tolerance.

- Large → easy detection, more noise

- Small → strict geometry, cleaner results

## Q4. “Why does the detection suddenly look perfect with certain parameters?”

### ✅ Observed Good Result

You found a parameter set that cleanly detected only the parking lanes.

### ✅ Why it worked

Each parameter aligned with real scene geometry:

| Parameter | Effect |
| --- | --- |
| minLineLength ≈ 90 | Removed cracks and noise |
| angle_min/max | Removed horizontal curb edges |
| angle_pair_th = 4 | Enforced strong parallelism |
| wide pair gap | Avoided rejecting true lanes |
| low threshold | Ensured both sides of lane detected |

### 🔑 Insight

Classical vision works best when:

thresholds encode real-world physical constraints

## Q5. “Same parameters, different crop → different result. Why?”

### ❌ Problem

Cropping the image at different heights caused:

- different angles

- different pixel gaps

- different visible line lengths

But parameters were absolute pixel values.

### ✅ Solution

Three fixes:

#### 1. Use a fixed ROI

Instead of arbitrary manual crops:

roi = img[int(0.4 * H):, :]

#### 2. Scale parameters relative to ROI size

minLineLength = 0.4 * ROI_height

min_pair_gap  = 0.1 * ROI_width

max_pair_gap  = 0.4 * ROI_width

#### 3. Rely more on geometry than spacing

- Keep angle_pair_th tight

- Keep gap range wider

### 🔑 Insight

Absolute pixel thresholds are fragile.
Relative geometry is robust.

## Q6. “Is there a principled way to eliminate crop & perspective sensitivity?”

### ✅ Long-Term Solution

Use Inverse Perspective Mapping (Bird’s-Eye View):

- Warp ground plane to top-down view

- Lane spacing becomes constant

- Lane angles stabilize

- Same parameters work everywhere

This is why real automotive systems use IPM.

### 🔑 Insight

Perspective is the root cause of most instability in monocular vision.

## Q7. “Why did debugging take so many iterations?”

### ✅ Because classical CV is constraint-driven, not data-driven

You were discovering:

- which constraints matter

- which ones are redundant

- which ones are scene-dependent

This is normal and valuable.

### 🔑 Meta Insight

You didn’t “tune parameters randomly” — you reverse-engineered the geometry of the scene.

That’s exactly how strong vision engineers think.

## 🧠 Final Mental Model

Your final detector works because it enforces:

- Long lines

- Correct orientation

- Strong parallelism

- Reasonable spacing

- Consistent geometry across frames

Each bug you hit was one of these constraints being:

- missing

- too strict

- or expressed in the wrong coordinate system

## ✅ What You’ve Achieved

By the end of this debugging process, you now have:

- A modular CV pipeline

- A principled lane-pair detector

- Intuition for parameter sensitivity

- A clear path to bird’s-eye view & parking slot extraction

This is very solid work, especially for a classical (non-DL) approach.

If you want, the next natural step is:

- grouping lane pairs into parking slots

- computing slot center + orientation

- feeding that into your parking planner